`

ATCSMU100 Design Specification & Integration on AE350

| Document Number | DSP-xxxxx |
| --- | --- |
| Date Issued | 2019-05-06 |
| Status | In-progress |

CONFIDENTIAL

# Revision History

| Rev. | Revision Date | Revised Chapter-Section | Revised Content | Author |
| --- | --- | --- | --- | --- |
| 0.6 | 2019-05-06 |  |  | Chevy |

| Review History |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev. | Review Date | Reviewed Chapter-Section | Comments | Reviewer |
| 0.2 | 2019-04-18 |  |  |  |

| Document Sign-off List |  |  |  |
| --- | --- | --- | --- |
| Department | Name | Date | Comment |
| VLSI |  | 2019-04-dd |  |

Table of Contents

Revision History	ii

List of Tables	vii

List of Figures	viii

1.	Overview	1

1.1.	Terminologies	1

1.2.	Features	1

1.3.	Block Diagram	3

1.4.	Function Description	3

1.4.1.	Software Programming Interface	3

1.4.2.	Power Control Slot	3

1.4.3.	System Control	4

2.	Design Configuration	5

3.	Signal Description	6

3.1.	Signal Definition	6

3.2.	Interface Protocol	9

4.	Micro-Architecture	12

4.1.	FSM (Moore)	12

5.	Programming Model	16

5.1.	Summary of Registers	16

5.2.	Register Description	17

5.2.1.	SYSTEMVER	17

5.2.2.	SYSTEMTS	17

5.2.3.	SMUVER	17

5.2.4.	SYSTEMCFG	18

5.2.5.	SYSTEMCR – SYSTEM clock ratio	18

5.2.6.	HARTn_RESET_VECTOR - CPUn Core reset vector (n: 0~3)	18

5.2.7.	PCS_SEL - Power control slot select	19

5.2.8.	PCS_CFG - Slot Configuration Register	19

5.2.9.	PCS_SCRATCH - Scratch pad	19

5.2.10.	PCS_MISC	19

5.2.11.	PCS_MISC2	20

5.2.12.	PCS_WE - Wakeup enable	21

5.2.13.	PCS_CTL - Power control slot control	21

5.2.14.	PCS_STATUS - Power control slot status	22

5.3.	PCS operation sequence	24

5.3.1.	Before an new operation	24

5.3.2.	PCS Reset operation	24

5.3.3.	PCS LightSleep operation	24

5.3.4.	PCS DeepSleep operation	24

5.3.5.	PCS Wakeup operation	25

5.3.6.	PCS Wakeup operation (in WAIT_STBY state)	25

5.3.7.	PCS DVFS operation (TBD)	25

6.	AE350 Integration	26

6.1.	Power domain	27

6.2.	Power state table	28

6.3.	Interface connection	28

6.4.	Software programming sequence	32

6.4.1.	The possible control flows for a Hart	32

6.4.2.	Example: Core 0 asks Core 1 to deep sleep mode	32

6.4.3.	Example: Cluster executes frequency change (DVFS)	33

7.	Schedule	33

7.1.	Issues	34

7.1.1.	Enhancement	34

7.1.2.	Resolved issues	34

7.1.3.	Pending issues	35

8.	Reference	37

# List of Tables

Table 1. Design Configuration	5

Table 1. Design Configuration	5

Table 2. Signal Definition	6

Table 3. State transition table	13

Table 4. State transition table (with output control)	13

Table 5. Register Summary	16

Table 6. Power domain partition	27

Table 7. Design Configuration	28

Table 8. List of wakeup event	28

# List of Figures

Figure 1. SMU Block Diagram	3

Figure 2. PCS Block Diagram	4

Figure 3. Logic Symbol	6

Figure 4. Reset	9

Figure 5. LightSleep & Wakeup	9

Figure 6. DeepSleep & Wakeup	10

Figure 7. DVFS-High to Low to High (DVFS_SUPPORT)	11

Figure 8. FSM	15

Typographical Convention Index

| Document Element | Font | Font Style | Size | Color |
| --- | --- | --- | --- | --- |
| Normal | Georgia | Normal | 12 | Black |
| Code | Lucida Console | Normal | 11 | Indigo |
| USER VARIABLE | Lucida Console | ALL-CAPS | 11 | INDIGO |
| Note/Warning | Georgia | Normal | 12 | Red |
| Hyperlink | Georgia | Bold + Underlined | 12 | Blue |

Overview

The SMU requirements are including:

- Register for system control

- System reset control

- Dynamic voltage frequency scaling (DVFS) support (TBD)

Terminologies

- SMU: System Management Unit

- PCS: Power Control Slot

- PD: Power Domain

- WFI: Wait for interrupt instruction

- DVFS: Dynamic voltage frequency scaling

- PLS: Partial Light Sleep (TBD)

Features

Power Domain Reset

  - A.K.A software reset

DVFS (TBD)

  - Purpose: Dynamic change power & frequency for workload requirement

  - Function: Change voltage and frequency

  - Latency: For voltage/clock source, it might take ms scale for scaling and stable.

Light Sleep

  - Purpose: Fast response for interrupt and save power

  - Function: PowerDomain are clock-gated

  - Latency: fast for enabling clock switch

Deep Sleep

  - Purpose: shut down the component to save most power

  - Function: PowerDomain are powered off.

  - Latency: For voltage/clock source, it might take ms scale for turn-on and stable.

Wakeup

  - Wakeup from Sleep modes by interrupts or SMU command

  - Maskable wakeup event

Cancel PCS operations from waiting (TBD)

Timeout detection (TBD)

Block Diagram

Programming Interface

Power Control Slot

Power Control Slot

Power Control Slot

Power Control Slot

Power control interface

Standby Interface

Wakeup Events

System bus interface

SMU interrupt

Wakeup

System

Control

ATCSMU100

Sys Reg

Sys Reg

Sys Reg

Sys Reg

Sys Reg

Figure 1. SMU Block Diagram

Function Description

Describe the function of each component of the design.

Software Programming Interface

The software programming interface is an APB slave device. It contains APB slave interface, and architecture registers for system and PCS operations.

Power Control Slot

- PCS controller

  - Reset

  - Clock

  - Power

  - Isolation

  - Retention

PowerDomain_n

Power-Gated

Cells

Power Control Slot

PCS_n

Registers

PCS_n

Controller

PowerController

ClockGenerator

ResetController

State

Retain

Iso

SoftwareProgramInterface

1

2

3

7

44

54

64

4. Voltage scaling

5. Frequency scaling

6. Reset control

7. Always-on power

8. Gated power

9. Gated clock

10. Reset

87

987

10987

1. Wakeup/Interrupt

2. Standby interface

3. Iso/Reten control

Figure 2. PCS Block Diagram

System Control

- Core_reset_vectors

- system_clock_ratio

  - sysclk_sel

  - bus_ratio

Design Configuration

Describe the hardware configurations of the design. Table 1 shows configuration macros.

Table 1. Design Configuration

| Configuration | Macro Name | Description |
| --- | --- | --- |
| PCS number | PCS_NUM | 1~8 PCS0 is for Always-on power domain |
| batch mode | PCS_BATCH_SUPPORT | Support PCS batch mode. (TBD) |
| DVFS mode | DVFS_SUPPORT | PCS support DVFS control (TBD) |
|  | FIX_VOLTAGE | Mask the voltage control on DVFS (for FPGA) |

Table 2. Design Configuration

| Configuration | Macro Name | Description |
| --- | --- | --- |
| timestamp |  | SYS_TS |  | 32-bit system timestamp number. = `AE350_BOARD_ID? |  |
|  |  | NDS_NHART |  |  |  |
|  | PRODUCT_ID |  |

Signal Description

Signal Definition

SMU

APB program IF

wakeup_events

pcsn_frq_scale_req

pcsn_frq_scale

pcsn_frq_clkoff

pcsn_frq_scale_ack

pcsn_vol_scale_req

pcsn_vol_scale

pcsn_vol_scale_ack

pcsn_iso

pcsn_reten

pcsn_int

pcsn_wakeup

pcsn_standby_req

pcsn_standby_ok

cpu_reset_vector0

cpu_reset_vector1

cpu_reset_vector2

cpu_reset_vector3

bus_ratio

pcsn_mem_init

pcsn_reset_n

pcsn_reset_source

(TBD)Clk/reset/test

sysclk_sel

Figure 3. Logic Symbol

Table 3 gives the detailed descriptions of I/O signals.

Table 3. Signal Definition

| Signal Name | I/O | Description |
| --- | --- | --- |
| SMU global signals |  |  |
| wakeup_event[31:0] | I |  |
| PCLK. PRESET_N. Psel. Peanble. Pwrie. PADDR[8:2]. PRDATA[31:0]. PWDATA[31:0]. pready. pslverr. | I/O | APB Interface |
| cpu_reset_vector0[31:0] cpu_reset_vector1[31:0] cpu_reset_vector2[31:0] cpu_reset_vector3[31:0] | O | HARTn_RESET_VECTOR RTL: output [127:0] core_reset_vectors; |
| bus_ ratio[2:0] | O | SYSTEMCR.bus_ratio |
| sysclk_sel[1:0] | O | SYSTEMCR.sysclk_sel |
| Power Control Slot signals (n from 0 to PCS_NUM) |  |  |
| pcsn_int | O | Interrupt (pcs_status.pend_int) |
| pcsn_wakeup | O | Wakeup interrupt (pcs_status.wakeup_int) |
| pcsn_standby_req | O | A hint to indicate the PCS is going to stop the clock |
| pcsn_standby_ok | I | The PD is ready for power change (on standby state) |
| pcsn_frq_scale_req | O | Frequency scaling request |
| pcsn_frq_scale[2:0] | O | Frequency scaling vector to clock generator. 0: clock gating 1: level 1 (lowest) 2: level 2 … (DVFS_SUPPORT) |
| pcsn_frq_clk_off[n:0] | O | Reserved for PLS |
| pcsn_frq_scale_ack | I | Frequency scaling acknowledge from clock generator 0: non stable 1: stable |
| pcsn_vol_scale_req | O | Voltage scaling request |
| pcsn_vol_scale[2:0] | O | Voltage scaling vector to voltage regulator. 0: power off (Power region shut-off indicator) 1: level 1 (lowest) 2: level 2 … (DVFS_SUPPORT) |
| pcsn_vol_scale_ack | I | Voltage scaling acknowledge from voltage regulator 0: non stable 1: stable |
| pcsn_iso | O | Isolation indicator |
| pcsn_reten | O | Retention indicator |
| pcsn_reset_n | O | Power region reset indicator |
| pcsn_mem_init[3:0] | O | [0]: MEM0 Reset Initialization [1]: MEM1 Reset Initialization [2]: MEM2 Reset Initialization [3]: MEM3 Reset Initialization |
| pcsn_reset_source[4:0] | I | [0]: power on reset occurs [1]: PCS-reset reset occurs [2]: reset source 1 occurs (AE350: Watchdog reset) [3]: reset source 2 occurs (AE350: Hardware reset) [4]: reset source 3 occurs (AE350: MPD power on reset) |

Interface Protocol

PCS-Reset operation waveform is shown in Figure 4.

The reset_n is deasserted after one cycle of the ST_Reset.

Figure 4. Reset

LightSleep & Wakeup operation waveform is shown in Figure 5.

The pcs_clk_req and pcs_clk_ack are pseudo-signals. Those signals are composed of pcs_frq_* and the clock state.

Figure 5. LightSleep & Wakeup

DeepSleep & Wakeup operation waveform is shown in Figure 6.

The pcs_clk_req and pcs_clk_ack are pseudo-signals. Those signals are composed of pcs_frq_* and the clock state.

The pcs_pwr_req and pcs_pwr_ack are pseudo-signals. Those signals are composed of pcs_vol_* and the power state.

Figure 6. DeepSleep & Wakeup

DVFS operation waveform is shown in Figure 7.

Figure 7. DVFS-High to Low to High (DVFS_SUPPORT)

Micro-Architecture

Describe the details of the design in this chapter.

FSM (Moore)

States

- ACTIVE

  - CLK/PWR=0n/on

- RESET (RST)

  - CmdReset for ResetCycle

  - DeepSleep for fixed cycles

  - pcsn_reset_n  ResetController

- WAIT_STBY

  - pcsn_standby_req is asserting and waiting for pcsn_standby_ok

  - The PD must assert the pcs_standby_ok within TimeoutCycle

    - else PCS will go to ACTIVE and assert interrupt

  - PCS command can be cancelled on this state

    - PCS will go to ACTIVE

- FRQ (CLK)

  - Frequency change on this state

  - pcsn_frq_scale_req  ClockGenerater

  - ClockGenerater  pcsn_frq_scale_ack

    - TODO: Timeout?

  - pcs_wakeup is asserted after the frequency change except gating clock

- VOL (PWR)

  - Voltage change on this state

  - pcsn_vol_scale_req PowerController

  - PowerController  pcsn_vol_scale_ack

    - TODO: Timeout?

- ISO

  - pcsn_iso=1 for IsoCycle

- RETEN

  - pcsn_reten=1 for RetenCycle

- LIGHT_SLEEP

  - CLK/PWR=0ff/on

- DEEP_SLEEP

  - CLK/PWR=0ff/off

Table 4. State transition table

| Input \ CurrentState | ACT | RST | WAI | CLK | LSL | ISO | RET | PWR | DSL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CmdReset | RST |  |  |  |  |  |  |  |  |
| CmdDeepSleep CmdLightSleep | WAI |  |  |  |  |  |  |  |  |
| CmdDVFSUp CmdDVFSDown | TBD |  |  |  | TBD |  |  |  | TBD |
| CmdCancel |  |  | ACT |  |  |  |  |  |  |
| pcsn_standby_ok |  |  | CLK |  |  |  |  |  |  |
| pcsn_clk_ack (pcsn_frq_scale_ack) |  |  |  | (F)LSL (F)ISO (R)WAI |  |  |  |  |  |
| pcsr_pwr_ack (pcsn_vol_scale_ack) |  |  |  |  |  |  |  | (F)DSL (R)RST |  |
| wakeup_event CmdWakeup |  |  |  |  | CLK |  |  |  | PWR |
| cnt=ResetCycle |  | ACT PWR RET |  |  |  |  |  |  |  |
| cnt=IsoCycle |  |  |  |  |  | RET CLK |  |  |  |
| cnt=RetenCycle |  |  |  |  |  |  | RST ISO |  |  |
| cnt=TimeoutCycle |  |  | ACT | TBD |  |  |  | TBD |  |
| 1 cycle (bypass) |  |  | ACT |  |  |  |  |  |  |

Table 5. State transition table (with output control)

| CurrentState | Input | NextState | Output |
| --- | --- | --- | --- |
| ACTIVE | CmdReset | RESET | pcs_reset_n=0 |
|  | Cmd*Sleep | WAIT | pcsn_standby_req=1 |
| RESET (RST) | cnt=ResetCycle (CmdReset) | ACT | pcs_reset_n=1 PCS_STATUS=Reset.PCS_reset |
|  | cnt=ResetCycle (constant 1) (CmdDeepSleep) | PWR | pcsn_pwr_req=0 |
|  | cnt=ResetCycle (constant 1) (FromDeepSleep) | RET | pcs_reten=0 |
| WAIT_STBY | CmdCancel | ACT | pcs_int=1 (?) |
|  | wakeup_event=0 pcsn_standby_ok=1 (Cmd*Sleep) | CLK | pcsn_clk_req=0 |
|  | 1 cycle (From*Sleep) | ACT | pcs_wakeup=1 PCS_STATUS = Active.wakeup_event_vector |
|  | wakeup_event=1 pcsn_standby_ok=1 (Cmd*Sleep) | ACT | pcs_wakeup=1 PCS_STATUS = Active.wakeup_event_vector |
|  | cnt=TimeoutCycle | ACT | pcs_int=1 |
| FRQ (CLK) | pcsn_clk_ack=0 (CmdLightSleep) | LIGHT_SLEEP | PCS_STATUS=Sleep.LightSleep |
|  | pcsn_clk_ack=0 (CmdDeepSleep) | ISO | pcs_iso=1 |
|  | pcsn_clk_ack=1 (From*Sleep) | ACT | pcs_wakeup=1 (pcsn_standby_req=0) PCS_STATUS = Active.wakeup_event_vector |
|  | pcsn_clk_ack=1 (From*Sleep) | WAIT_STBY | pcsn_standby_req=0 |
| LIGHT_SLEEP | wakeup_event CmdWakeup | CLK | pcsn_clk_req=1 |
| ISO | cnt=IsoCycle (CmdDeepSleep) | RET | pcs_reten=1 |
|  | cnt=IsoCycle (FromDeepSleep) | CLK | pcsn_clk_req=1 |
| RETEN | cnt=RetenCycle (CmdDeepSleep) | RST | (delay 1 cycle) pcs_reset_n=0 |
|  | cnt=RetenCycle (FromDeepSleep) | ISO | pcs_iso=0 |
| VOL (PWR) | pcsn_pwr_ack=0 (CmdDeepSleep) | DEEP_SLEEP | PCS_STATUS = Sleep.DeepSleep |
|  | pcsn_pwr_ack=1 (FromDeepSleep) | RST | pcs_reset_n=1 |
| DEEP_SLEEP | wakeup_event CmdWakeup | PWR | pcsn_pwr_req=1 |

Figure 8. FSM

Programming Model

Summary of Registers

Table 6 provides a summary of registers.

Table 6. Register Summary

| Address | Description | Type | Reset |
| --- | --- | --- | --- |
| ID, revision and configuration register |  |  |  |
| +0x00 | SYSTEMVER | RO |  |
| +0x04 | SYSTEMTS | RO | - |
| +0x08 | SYSTEMCFG | RO |  |
| +0x0c | SMUVER | RO |  |
| Global control registers |  |  |  |
| 0x10 | WRSR Wake-up and reset status register |  |  |
| 0x14 | SMUCR SMU command register |  |  |
| 0x1C | WRMASK Wake-up and reset mask register |  |  |
| 0x20 | CER Clock enable register |  |  |
| 0x24 | SYSTEMCR – SYSTEM clock ratio | RW | - |
| 0x40 | SCRATCH Scratch pad register |  |  |
| +0x50~0x5c | HARTn_RESET_VECTOR - CPUn Core reset vector (n:0~3) | RW |  |
| RESERVED | PCS_SEL - Power control slot select (PCS_BATCH_SUPPORT) | RW |  |
| Power Control Slot registers (PCS0 or PCS batch mode) |  |  |  |
| +0x80 | PCS_CFG - Slot Configuration Register | RO |  |
| +0x84 | PCS_SCRATCH - Scratch pad | RW |  |
| +0x88 | PCS_MISC | RW |  |
| +0x8c | PCS_MISC2 (reserved for partial light sleep) | RW |  |
| +0x90 | PCS_WE - Wakeup enable | RW |  |
| +0x94 | PCS_CTL - Power control slot control | RW |  |
| +0x98 | PCS_STATUS - Power control slot status | RW |  |
|  | Power Control Slot registers (PCS1 to PCSn) |  |  |
| +(0x20*n+0x80) | PCS_CFG - Slot Configuration Register | RO |  |
| +(0x20*n+0x84) | PCS_SCRATCH - Scratch pad | RW |  |
| +(0x20*n+0x88) | PCS_MISC | RW |  |
| +(0x20*n+0x8c) | PCS_MISC2 (reserved for partial light sleep) | RW |  |
| +(0x20*n+0x90) | PCS_WE - Wakeup enable | RW |  |
| +(0x20*n+0x94) | PCS_CTL - Power control slot control | RW |  |
| +(0x20*n+0x98) | PCS_STATUS - Power control slot status | RW |  |

Register Description

Describe the details of all registers.

SYSTEMVER

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
|  |  | See AndesCore_x25x_DS for detail. | RO |  |

SYSTEMTS

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
|  |  | timestamp for development | RO | SYS_TS |

SMUVER

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
|  |  |  | RO |  |

SYSTEMCFG

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
|  |  | See AndesCore_x25x_DS for detail. | RO |  |

SYSTEMCR – SYSTEM clock ratio

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| sysclk_sel | 1:0 (TODO:[0]) | System clock select 0: OSCH 1: Divide OSCH by 2 2~: reserved | RW | 0 |
| bus_ratio | 4:2. (TODO:[3:1]) | Bus clock ratio select. Value; aclk:hclk:pclk frequency. 0; 1:1:1. 1; 1:1:1/2. 2; 1:1:1/4. 3; 1:1/2:1/2. 4; 1:1/2:1/4. (SMU v1.0) aclk:core_clk=1:1 | RW | 0 |

HARTn_RESET_VECTOR - CPUn Core reset vector (n: 0~3)

See AndesCore_x25x_DS for detail.

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| reset_vector | 31:0 | Entry address upon processor reset [1:0]: hardwire 0. | RW | 0x80000000 `AE350_SMU_RESET_VECTOR_DEFAULT |

PCS_SEL - Power control slot select

IM Requirement: PCS_BATCH_SUPPORT = 1

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| pcs0_sel | 0 | Select Power control slot 0 | RW | 0x0 |
| pcs1_sel | 1 | Select Power control slot 1 | RW | 0x0 |
| pcsn_sel | n | Select Power control slot n | RW | 0x0 |

PCS_SEL[7:0] = 8’b0110, select PCS1 and PCS2.

PCS_CFG - Slot Configuration Register

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| capability | 5:0 | Slot control capability [0]: Reset [1]: DVFS (hardwire 0) [2]: LightSleep [3]: DeepSleep [4]: Cancel from waiting (hardwire 0) [5]: Timeout detection (hardwire 0) | RO |  |

PCS_SCRATCH - Scratch pad

This is a scratch register, which retains values when the rest of the system is powered down. It could be used to hold some parameters during the power down period.

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| SCRATCH | [31:0] | Scratch register | RW | 0 |

PCS_MISC

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| iso_cyc | 3:0 | ISO cycle – ST_ISO cycles (For DeepSleep) 0: 1 cycle 1: 1 cycle 2: 2 cycles … | RW | 0xf |
| reten_cyc | 11:4 | RETEN cycle – ST_RET cycles (For DeepSleep) 0: 1 cycle 1: 1 cycle 2: 2 cycles … | RW | 0xff |
| to_cyc | 27:12 | (Wait standby) Timeout cycle (x16) 0: No timeout 1: 16 cycles 2: 32 cycles … | RW | 0 |
| mem_init0 | 28 | MEM0 Reset Initialization 0: 1: The Reset Initialization is required on MEM0 | RW/RO* | 1/0 |
| mem_init1 | 29 | MEM1 Reset Initialization | RW/RO* | 1/0 |
| mem_init2 | 30 | MEM2 Reset Initialization | RW/RO* | 1/0 |
| mem_init3 | 31 | MEM3 Reset Initialization | RW/RO* | 1/0 |

Note: RO and hardwire 0 when the field is not supported in the individual PCS.

PCS_MISC2

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
|  |  | Reserved for PLS |  |  |

PCS_WE - Wakeup enable

This register controls the wakeup event for selected power domain.

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| wakeup_en | [31:0] | bit[n]=1: Enable corresponding wakeup event[n] [0]: wakeup by PCS wakeup command. (hardwire1) | RW | 0xffffffff |

PCS_CTL - Power control slot control

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| cmd | 2:0 | Power control Command. 0: Active 1: PCS_reset* 2: DVFS 3: Sleep 4~7: reserved | RW* | 0 |
| param | 7:3 | cmd; param. Active; 0: reserved 1: wakeup 2: Cancel from waiting state. PCS_reset; minimal Reset cycles 0: reserved. DVFS; DVFS factor (level) 0: highest frequency 1: Reserved 2: Reserved … 7: lowest frequency. Sleep; 0: LightSleep 1: DeepSleep. | RW* | 0 |

Note:

Hardware protection on Write operation:

- When the PCS is in Busy_Wait state, the Write operation is waived and it can be detected in PCS_STATUS.cmd_status.

- When the PCS is not in Busy_Wait state, the PCS_CTL will be updated. An interrupt occurs when the command is not supported or is illegal on current state. The wrong PCS command can be detected in PCS_STATUS.cmd_status.

Read operation: The field will be cleared after the command finishes.

Q: global reset by PCS0? YES

Except AOPD elements

PCS_STATUS - Power control slot status

| Name | Bit | Description | Type | Reset |
| --- | --- | --- | --- | --- |
| PD_type | 2:0 | 0: Active 1: Reset 2: Sleep 3: Busy_Wait 7: Timeout | RW* | 1 |
| PD_status | 7:3 | PD_type; PD_status. Active; Wakeup cause. Wakeup from non-masked pending event (vector). 0: Wakeup from PCS or DVFS done 1: wakeup event 1…. Reset; Reset cause: [3]: global power-on reset [4]: PCS_reset (SW reset in SMU v0) [5]: implementation dependent [6]: implementation dependent [7]: implementation dependent. Sleep; Sleep mode: 0: LightSleep 1~15: reserved for partial light sleep. 16: DeepSleep 17~31: reserved. Busy_Wait; 0: waiting on Reset 1: waiting on DVFS 2: waiting on LightSleep 3: waiting on DeepSleep 16: Busy on Reset 17: Busy on DVFS 18: Busy on LightSleep 19: Busy on DeepSleep. Timeout; 0: Reset Timeout 1: DVFS Timeout 2: LightSleep Timeout 3: DeepSleep Timeout Wakeup Timeout. | RW* | 0 |
| cmd_status | 10:8 | 0: Command successful 1: Cancel successful (Command unsuccessful) 6: waive PCS command ( no pend_int) 7: Unsupported/Wrong PCS command (fire pend_int) | RW | 0 |
| DVFS | TBD | DVFS factor | RO | 0 |
| wakeup_int | 30 | Pending wakeup interrupt. The bit is asserted when the PD_type is changed to Active by PCS. | W1C | 0 |
| pend_int | 31 | Pending interrupt. The bit is asserted when the PD_type is changed to Timeout by PCS or the cmd_status is 7. | W1C | 0 |

*Note:

Write 0 Clear, when status is stable (PD_Type != Busy_Wait)

Keep the latest value.

PCS operation sequence

Before an new operation

  - PCS_SEL (batch mode)

  - Check/Clear status/interrupts

  - Initialize PCS_WE, PCS_MISC …

PCS Reset operation

  - Write PCS_CTL

    - cmd = PCS_reset

    - param = Reset cycle count

  - The reset will be asserted for specific cycles

  - The PD goes to ACTIVE state.

    - PCS_STATUS. PD_Type = Reset

    - PCS_STATUS. PD_Status = PCS_reset

PCS LightSleep operation

  - Write PCS_CTL

    - Sleep.LightSleep

  - PCS goes to ST_Wait_Standby

    - Pcs_standby_req is asserting and waiting for pcs_standby_ok

  - The clock is gated after the pcs_standby_ok asserted

  - The PD goes to LightSleep state.

    - PCS_STATUS = Sleep.LightSleep

PCS DeepSleep operation

  - Write PCS_CTL

    - Sleep.DeepSleep

  - PCS goes to ST_Wait_Standby

    - Pcs_standby_req is asserting and waiting for pcs_standby_ok

  - The clock will be gated after the pcs_standby_ok asserted

  - The isolation/retention cells are enable

  - The reset is asserted and then the power is gated

  - The PD goes to DeepSleep state.

    - PCS_STATUS = Sleep.DeepSleep

PCS Wakeup operation

  - The PD is under DeepSleep/LightSleep state

  - Wakeup occurs

    - Wakeup by PCS

      - PCS_CTL = Active.wakeup

    - Non-masked wakeup event occurs

  - (Wakeup from DeepSleep) The power is supplied and then the reset is de-asserted

  - (Wakeup from DeepSleep) The isolation/retention cells are disabled and the state is recovered.

  - The clock is generated

  - The PD goes to Active state

    - PCS_STATUS. PD_Type = Active

    - PCS_STATUS. PD_Status = wakeup event vector

    - pcs_wakeup is asserted

PCS Wakeup operation (in WAIT_STBY state)

The PD can be wakeup immediately from ST_WAIT_STBY.

  - The PD is under WAIT_STBY state

    - Pcs_standby_req is asserting and waiting for pcs_standby_ok

  - Wakeup occurs

    - Wakeup by PCS

      - PCS_CTL = Active.wakeup

    - Non-masked wakeup event occurs

  - The PD goes to Active state after the pcs_standby_ok asserted

    - PCS_STATUS. PD_Type = Active

    - PCS_STATUS. PD_Status = wakeup event vector

    - pcs_wakeup is asserted

PCS DVFS operation (TBD)

AE350 Integration

- atcsmu100.v

- ae350_pin.v

  - ae350_aopd_pin.v

- ae350_rstgen.v

  - ae350_aopd_rstgen.v

- ae350_clkgen.v

  - ae350_aopd_clkgen.v

- ae350_aopd_testgen.v

  - (merge) ae350_testgen.v

- ae350_vol_ctrl.v

Power domain

Table 7. Power domain partition

| PD | Description/Components | power state |
| --- | --- | --- |
| Always on (AO) (PCS0) | SMU/RTC ncejdtm200.ncejdtm200_tap ae350_clkgen ae350_rstgen.v ae350_pin.v ae350_testgen.v (next revision) ae350_aopd_clkgen ae350_aopd_rstgen.v ae350_aopd_pin.v | ON |
| Platform0 (PCS1) | PLDM/ncejdtm200 | ON (+LightSleep) OFF(DeepSleep) |
| Platform1 (PCS2) (MPD) | BUSes/PIPs / PLICs/PLMT … | ON(+LightSleep) OFF(DeepSleep) |
| CPU0~3 (PCS3~6) | Core_top_0+L2C Core_top_1 Core_top_2 Core_top_3 defparam smu_pcs3.PCS_RESET_SUPPORT = 0; see A25MP_DS: HART_RESET_CTL (0x44) | HI(+LightSleep) LO(+LightSleep) OFF(DeepSleep) |

defparam smu_pcs0.LIGHT_SLEEP_SUPPORT = 0;

defparam smu_pcs0.DEEP_SLEEP_SUPPORT = 0;

defparam smu_pcs3.PCS_RESET_SUPPORT = 0; // core0 cannot be reset by PCS

defparam smu_pcs3.FIELD_MEM_INIT = 2;

defparam smu_pcs4.FIELD_MEM_INIT = 2;

defparam smu_pcs5.FIELD_MEM_INIT = 2;

defparam smu_pcs6.FIELD_MEM_INIT = 2;

Power state table

The legal combinations of states of each power domain. (UPF view, +DVFS)

Table 8. Design Configuration

| Level | Platform 0 | Platform 1 | CPU 0 | CPU 1 | CPU 2 | CPU 3 | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | ON/- | - (OFF) | - | - | - | - |  |
| 1 | ON/- | ON | - | - | - | - |  |
| 2 | ON/- | ON | HI/LO | - | - | - | * |
| 3 | ON/- | ON | HI/LO | HI/LO | - | - | * |
| 4 | ON/- | ON | HI/LO | HI/LO | HI/LO | - | * |
| 5 | ON/- | ON | HI/LO | HI/LO | HI/LO | HI/LO | * |

Note: Depends on the L2C implementation, CPUn may be in the same power on state.

Currently, L2C & CPU must run on the same frequency.

Interface connection

- wakeup_event[31:0]

  - CPU Power Domain

  - Platform Power Domain

Table 9. List of wakeup event

| NO. | Signal | Description |  |
| --- | --- | --- | --- |
| 0 | - | PCS wakeup command |  |
| 20:1 | int_src[20:1] | PLIC interrupt source |  |
| 21 | dbg_wakeup_req |  | ae350_debug_subsystem debug request (from ncejdtm200) |
| 22 | X_wakeup_in; | External wake-up event |  |
| 28 | debugint | from ae350_debug_subsystem PCS[default].wakeup_event.debugint = hart0_debugint PCS[CPU0].wakeup_event.debugint = hart0_debugint PCS[CPU1].wakeup_event.debugint = hart1_debugint PCS[CPU2].wakeup_event.debugint = hart2_debugint PCS[CPU3].wakeup_event.debugint = hart3_debugint |  |
| 29 | sip | PLIC_SW interrupt target PCS[default].wakeup_event.msip = hart1_msip PCS[CPU0].wakeup_event.msip = hart0_msip / 0 PCS[CPU1].wakeup_event.msip = hart1_msip PCS[CPU2].wakeup_event.msip = hart2_msip PCS[CPU3].wakeup_event.msip = hart3_msip |  |
| 30 | tip | PLMT PCS[ default ].wakeup_event.mtip = hart0_mtip PCS[CPU0].wakeup_event.mtip = hart0_mtip PCS[CPU1].wakeup_event.mtip = hart1_mtip PCS[CPU2].wakeup_event.mtip = hart2_mtip PCS[CPU3].wakeup_event.mtip = hart3_mtip |  |
| 31 | eip | PLIC interrupt target (*=m/s/u) PCS[ default ].wakeup_event.eip = hart0_*eip PCS[CPU0].wakeup_event.eip = hart0_*eip PCS[CPU1].wakeup_event.eip = hart1_*eip PCS[CPU2].wakeup_event.eip = hart2_*eip PCS[CPU3].wakeup_event.eip = hart3_*eip |  |

List of PLIC interrupt source (ae350_chip.v 1.113+)

int_src[1] = rtc_int_period; // = rtc_int_hsec ¦ rtc_int_sec ¦ rtc_int_min ¦ rtc_int_hour ¦ rtc_int_day; int_src[2] = rtc_int_alarm; int_src[3] = pit_intr; int_src[4] = spi1_int; int_src[5] = spi2_int; int_src[6] = i2c_int; int_src[7] = gpio_intr; int_src[8] = uart1_int; int_src[9] = uart2_int; int_src[10] = dma_int; int_src[17] = ssp_intr; int_src[18] = sdc_int; int_src[19] = mac_int; int_src[20] = lcd_intr; int_src[23] = pcs_cpu0_standby_req; // target: cpu0 int_src[24] = pcs_cpu1_standby_req; // target: cpu1 int_src[25] = pcs_cpu2_standby_req; // target: cpu2 int_src[26] = pcs_cpu3_standby_req; // target: cpu3 int_src[27] = pcs_cpu0_wakeup; // target: cpu0 int_src[28] = pcs_cpu1_wakeup; // target: cpu1 int_src[29] = pcs_cpu2_wakeup; // target: cpu2 int_src[30] = pcs_cpu3_wakeup; // target: cpu3 int_src[31] = pcs_cpu0_int ¦ pcs_cpu1_int ¦ pcs_cpu2_int ¦ pcs_cpu3_int ¦ pcs_Platform0_int ¦ pcs_Platform1_int ¦ pcs_ao_int; // target: cpu0

- pcs_wakeup[n:o]

  - CPU Power Domain

    - int_src[x] = pcs_wakeup[y]

      - hartN is wakeup (terminate WFI) by PCS via ExternalInterrupt

      - (hartN can also be wakeup by hart0 via SoftwareInterrupt)

  - Platform Power Domain

    - (Floating)

      - Must de-assert pcs_standby_req (Hardware solution)

- pcs_int[n:o]

  - CPU Power Domain,

    - hart0 handle all of the pcs_int[]

      - int_src[x] = |pcs_int[y:z]

      - PLIC redirect the pcs_int[y:z] to hart0.MIE

    - (V0.1)

      - Merge pcs_wakeup & pcs_interrup

        - Hart0 check the status to identify wakeup event or other intrrupts

      - hart0 handle all of the pcs_int[]

      - int_src[x] = |pcs_int[y:z]

      - PLIC redirect the pcs_int[y:z] to hart0.MIE

      - hartN can be wakeup by hart0 via SoftwareInterrupt

    - hartN handle the pcs_int respectively

      - int_src[x] = pcs_int[y]

      - PLIC redirect the pcs_int to hartN.MIE respectively

  - Platform Power Domain,

    - hart0 handle all of the pcs_interrupts, or

      - int_src[x] = |pcs_int[y:z]

      - PLIC redirect the pcs_int[y:z] to hart0.MIE

    - PD handle the pcs_int respectively

      - pcs_int[y]

- pcs_standby_req[n:o]

  - (floating) Software solution, hart0 inform components to entering standby mode

    - CPU Power Domain, execute WFI on hart N via SoftwareInterrupt

    - Platform Power Domain, disable IPs

  - Hardware solution, standby request from PCS to inform components to entering standby mode

    - CPU Power Domain, execute WFI on hart N via ExternalInterrupt

      - int_src[x] = pcs_standby_req[y]

      - PLIC redirect the pcs_standby_req to hartN.MIE respectively

    - Platform Power Domain,

      - pcs_standby_req[y]

- pcs_standby_ok[n:o]

  - CPU Power Domain

    - pcs_standby_ok[x] = hart0_core_wfi_mode & l2c_idle (l2c_pcs_standby_ok)

    - pcs_standby_ok[x] = hart1_core_wfi_mode

    - pcs_standby_ok[x] = hart2_core_wfi_mode

    - pcs_standby_ok[x] = hart3_core_wfi_mode

  - Platform Power Domain

    - pcs_standby_ok[x] = &IP_idle[], or

    - pcs_standby_ok[x] = hardwire 1

      - Software solution, software guarantee the components are already in the standby mode

- pcs_mem_init[3:o]

  - CPU Power Domain

    - icache_disable_init = pcs_mem_init[o]

    - dcache_disable_init = pcs_mem_init[o] (or pcs_mem_init[1])

    - l2c_disable_init = pcs_mem_init[2]

Software programming sequence

The possible control flows for a Hart

Sleeps

  - PCS[cpu1].CmdSleeps PCS[cpu1]  pcs_standby_req[cpu1] cpu1[meip] (&meie)  CPU1.WFI  pcs_standby_ack PD[cpu1] clock off

  - PCS[cpu1].CmdSleeps & CPU1.WFI  pcs_standby_ack PCS[cpu1] PD[cpu1] clock off

Wakeup

  - PCS[cpu1].CmdWakeupPCS[cpu1] PD[cpu1] clock on

    - PCS[cpu1].wakeup cpu1[mei]  cpu1 exit WFI

    - (v0.1) PCS[cpu1].wakeup cpu0[mei] cpu0 fire cpu1[msi] cpu1 exit WFI

  - cpu1[mti/msi]PCS[cpu1] PD[cpu1] clock on

    - (v0.2) PCS[cpu1].wakeup cpu1[mei]  cpu1 exit WFI

    - (v0.1 & v0.2)  cpu1[mti/msi]  cpu1 exit WFI

Example: Core 0 asks Core 1 to deep sleep mode

  - Core 0 inform Core 1 to enter WFI/standby mode

  - Core 0 send power command to SMU for Core 1

  - SMU handles power command

  - Core 1 enters deep sleep mode

  - Core 1 wakeup by Core 0 by Software PLIC

Example: Cluster executes frequency change (DVFS)

  - Core 0 lock SMU access

  - Core 0 setup DVFS command to SMU

  - Core 0 inform other cores to execute WFI

  - Core 0 executes WFI

  - SMU wait all power domains are in standby mode

  - SMU handles DVFS command to all cores

  - PD_Core_0 is wakeup by SMU interrupt

    - SMU interrupt is automatically asserted by DVFS command

  - Core 0 send IPI to wakeup other cores

  - Core 0 unlock SMU access

Schedule

| Task | Description |  |  |
| --- | --- | --- | --- |
| Spec | Revision V0.1 and review |  |  |
|  | MicroArchSpec |  |  |
|  | ppt |  |  |
|  | SMU Revision V0.5 |  |  |
|  | DataSheet |  |  |
| Design |  |  |  |
| SMU | Interface Done |  |  |
| Ae350 | reset gen clk gen pwr gen (interface) SMU Connection |  |  |
| UPF/CPF | PD ISO RET (RAM) |  |  |
| Verification |  |  |  |
| Test plan |  |  |  |
| single core | - LightSleep (without UPF) - DeepSleep |  |  |
| multi core | - LightSleep (without UPF) - DeepSleep |  |  |

Deadline:

Issues

Enhancement

Different frequency between cores and L2

  - Support CORE:L2C/AXI = 1:2/2:1.

Currently, Caches must be flushed when clock off.

Core 0 and L2 are de-coupled in power domain.

Power domain inside SMU for ultra-low power

Add interface to quick wakeup from Bus_req

Resolved issues

- Debug when core power off before

Scenario

  - Core 0 setup trigger module

  - Core enter deep sleep mode

  - Core 0 wakeup by interrupt

In this case, Core 0 loses all setting in trigger module and OpenOCD and debug module can’t detect this situation.

Solution

  - Use retention cells or always-on region for debug CSRs (HW solution)

  - Do not backup debug CSRs and OpenOCD handles this issue

L2C design request for power mode

For light sleep (clock-gated) mode and DVFS

  - L2C should stop snooping when cores are executing WFI separated

    - L2 will stop snooping to the core when its’ output port “ready_for_snoop” is de-asserted. (from Jimmy)

  - L2C should enter standby mode automatically after all cluster are in WFI mode

    - Queue in L2C should be empty

    - L2 will add one output port “l2_idle” and it should be asserted when all transactions in L2 have been done. (from Jimmy)

  - Coherence slave port should be disable while DVFS is doing or L2C is in light sleep mode

    - Master should know that coherence slave port is temporarily unavailable.

    - There is no clock in L2 and perhaps we should handle this in system level by adding a wrapper. (from Jimmy)

In L2C standby mode

  - A output indicate that L2C is busy

    - SMU handles power change when L2C is not busy.

Debug mode

- Debug module supports bus access

  - Wakeup bus when debug_wakeup is asserted by debug module

    - In AE350 SMU design, enable main bus clock.

      - Currently(ae350_smu.v), pclk/hclk will be enabled By HW

  - Debug module wakeup a core by SMU interrupt

  - (Chevy)

    - Debug module in PD. Platform0

    - BUSes in PD. Platform1

- Debug module does not support bus access

  - Wakeup core 0 for debug

    - An additional wakeup source debug_int is required.

Pending issues

Interrupt occur during DVFS flow

- Disable all interrupt source

- Cons: Long latency for interrupt response

- (Chevy) Add new signal: pcs_standby_req_dvfs

  - Core cannot be wakeup

- Register read under PCS_BATCH_SUPPORT

- TBD

Note / Issue / Enhancement

| .dbg_wakeup_ack (dbg_wakeup_ack ), // (ae350_cpu_subsystem) <= (ae350_smu) .dbg_wakeup_req (dbg_wakeup_req ), // (ae350_cpu_subsystem) => (ae350_smu) smu_hclk_en_r <= 1'b1; smu_pclk_en_r <= 1'b1; bus clocks are enable when dbg_wakeup_req | ae350_smu.v |
| --- | --- |
| system.ae350_chip.ae350_smu.l2c_mask_req = &core_wfi_mode[3:0]; Q: function = pcs_standby_req? Enhance: control by l2c itself instead of by smu l2c_mask_req (inside L2C) = pcs_standby_req & core*_idle | E |
| ae350_smu.v smu_pclk_en = {clock_en_reg[10:3], smu_pclk_en_r}; v0.2只能disable whole PD clock。 不能disable individual IP!!! v0.3需要提供介面(參考mem_init)，讓clk_gen去處理?! partial light sleep. wait standby_ok Platform PD == Cpu PD waive standby_ok v0.3 Platform PD != Cpu PD | E |
| Synchronizer position? outside SMU | I |
| ae350_smu.v clocks pclk T_OSCH clk_32k (OSCL) ae350_smu_aopd_core.v | I |
| AE350_SMU_USERDR0_SUPPORT? // User define register 0 1 offset=0x30/0x34 | Remove |
| ahb2core_clken apb2core_clken apb2ahb_clken | ae350_clkgen.v |
| Clock Enable Register (CER) `AE350_SMU_CLKE_DEFAULT `AE350_SMU_PCLKE_DEFAULT, `AE350_SMU_HCLKE_DEFAULT, `AE350_SMU_CCLKE_DEFAULT |  |
| what happen when PD on reset and PCS on waiting/busy (scaling) |  |
| // main power power off signal // if define AE350_SMU_MDP_PWR_OFF_HIGH, // the power off command would set the X_mpd_pwr_off to high to power off main power // if don't define AE350_SMU_MDP_PWR_OFF_HIGH, // the power off command would set the X_mpd_pwr_off to low to power off main power `define AE350_SMU_MDP_PWR_OFF_HIGH | remove |
| // when X_wakeup_in high to trigger the wakeup event, must define the macro AE350_SMU_EXT_WAKEUP_HIGH //`define AE350_SMU_EXT_WAKEUP_HIGH | remove |
| PCS0_reset for global reset (was sw_rstn) |  |
| I/O Signals X_mpd_pwr_off O Main power domain power off indication (bus or CPU0 or CPU*) |  |
| TODO: 不回VOL/FRQ_ack怎辦? 要能cancel/timeout? |  |
| X_por_b I Power-on reset in the main power domain | reset_vector core_reset ALL |

AE350P design spec

"T:\project\Vicuna\Specification\Design\AE350\AE350_Design_Specification_V1.0.docx"

Reference